Session Report

PINN surrogate for a closely-spaced rod bundle (ENYGF, AI & Nuclear Symbiosis track)

24-25 September 2026  |  Repository AbdullahKurdi-1/ENYGF, branch claude/nuclear-ai-conference-proposal-5tz4xk

# 1. Summary

This session took the project from a converged CFD case to a working, validated training pipeline. In order:

- Understood the CFD training data and compared it point by point with both reference papers (Shams & Kwiatkowski 2018; Mathur et al. 2023 DNS).

- Found a clear physical pattern: the steady RANS CFD matches the DNS in the open subchannel but badly under-predicts turbulence and mixing in the narrow gap (no gap vortex street).

- Diagnosed the first full PINN training: the flow was learned, but every temperature field collapsed to T = 0, so its Nusselt numbers were meaningless.

- Rebuilt the CFD case independently, reproduced the failure and fixed it. On the test copy the PINN now reproduces the CFD Nusselt numbers within 1.5% and the pressure gradient within 0.2%.

- Added the CFD's own Nusselt numbers (defined exactly as in the DNS). Your real CFD gives iso-temperature Nu within 2-14% of the DNS and iso-flux Nu 30-45% low - a physically explained RANS result.

Status at the end of the session: CFD rerun and extracted successfully on your machine (new viscosity, full-field data, CFD Nusselt numbers). The corrected PINN training in the notebook is the next thing to run.

# 2. Questions you asked and the answers

Earlier in the session (set-up and first CFD runs):

Question: Is OpenFOAM 11 a downgrade from 13? What do steps 5 and 6 do - I expected ML in Jupyter.

Answer: We kept your OpenFOAM v13 and rewrote the case in v13 syntax (foamRun + incompressibleFluid, scalarTransport function object). The ML runs in a Jupyter notebook (ml/notebooks/pinn_walkthrough.ipynb); the same code can also run from the terminal.

Question: You changed something on GitHub - how does my terminal get it?

Answer: It is not automatic: run git pull in ~/ENYGF to download new commits.

Question: You ignored Ux and Uy - isn't that manipulating the data?

Answer: No. Only the convergence check skips columns that are zero to round-off (1e-15: k-omega SST produces no secondary flow), because a relative change of noise is meaningless. The data are untouched and the script prints which columns it skipped.

Question: Is my machine too weak? Should I use the university lab?

Answer: A laptop CPU is enough: a full training run is about 30-60 minutes. A lab GPU would only make it faster.

Question: Jupyter problems (old projects shown, cell stuck at [*]).

Answer: Windows Jupyter was holding port 8888; start the WSL one with jupyter notebook --port 8890 --no-browser. A cell stuck at [*] after switching kernels is fixed with Kernel > Restart.

Today (analysis and training):

Question: What are those? (the Step 1 training-data plots)

Answer: They are the CFD profiles along the DNS path xi (rod in the narrow gap -> gap centre at xi/Dh = 0.105 -> subchannel centre at 1.19 -> rod at 45 deg at 1.75). Velocity is low in the gap (0.55) and highest at the subchannel centre (1.38); eddy viscosity is near zero in the gap; TKE peaks near the walls. Temperatures are negative because the rod is the hottest point (sink in the fluid); low Pr (liquid metal) is flat and laminar-like, high Pr has thin, steep wall layers. In iso-flux the narrow-gap wall is hotter than the 45 deg wall - rods run hottest where they face each other.

Question: Compare these graphs to the two reference papers - where does it match and where not?

Answer: Matches in the open subchannel: centre velocity (1.39 vs DNS 1.36), TKE at xi/Dh = 0.6 and at the subchannel centre (within 6%), liquid-metal wall heat flux ratio (0.93 of DNS), gap velocity equal to the 2018 URANS (0.55). Does not match in the narrow gap: TKE about 10x too low, gap wall shear ratio 0.29 vs 0.69, gap velocity 0.55 vs about 0.85, Pr 1-2 gap temperature and wall-heat-flux ratios 40-60% of DNS. One cause explains all of it: steady RANS has no gap vortex street (the 3.7 Hz pulsations), so it cannot mix momentum and heat through the gap. Liquid metal is immune because conduction dominates. See Section 4.

Question: (Training results) - loss curves, PINN vs CFD plots and Nusselt bars.

Answer: The first training failed for temperature. Flow was learned well, but every PINN temperature curve was flat at about 0 (the network collapsed to the trivial T = 0 state), so the Nusselt bars (e.g. 322 for liquid metal) were meaningless and must not be reported. The learned dp/dz was 0.295 vs the CFD's 0.188 (+57%): the momentum equation was never satisfied. Causes and fixes are in Section 3.

Question: The extractor crashed: TypeError: 'NoneType' object is not subscriptable.

Answer: OpenFOAM v13 stores only the gradient of the iso-flux wall boundary, not the wall temperature. The extractor now rebuilds it as T_cell + gradient x wall distance, exactly how OpenFOAM evaluates it; checked to give identical Nusselt numbers.

Question: Step 1 table - CFD Nusselt numbers vs DNS.

Answer: Correct and identical to the extractor output. Liquid-metal iso-T matches the DNS (+2%); Pr 1-2 iso-T are 13-14% low; iso-flux 30-45% low because the gap wall runs hot. This is the RANS model's error; the PINN can only reproduce the CFD column.

# 3. Change log: every change, why, and its effect

Commits on the branch today, oldest first. Effects measured on your data where available, otherwise on the independent test copy (Section 5).

| Commit | Change | Why | Effect |
| --- | --- | --- | --- |
| 85eb696 | Added ml/src/compare_cfd_to_papers.py: prints CFD values at the gap centre, subchannel centre and both walls next to values read from the DNS (Figs 6, 7, 9a, 11a, 12a) and the 2018 URANS. | Needed exact CFD numbers instead of readings from a screenshot. | Produced the comparison in Section 4; confirmed the gap under-prediction quantitatively. |
| b51b6ae | Removed the 2018 'gap friction velocity = 0.045' reference. | On re-checking, 0.045 equals the 2018 paper's rod-averaged value (Re_tau/Re = 439/9800), not a gap value. | Avoids comparing against an invalid number. |
| 8ff9d1d | Training data: every CFD cell (cfd_field.csv) instead of 4 sampled lines, plus wall shear and iso-T wall heat flux per rod face; 20% of cells held out for testing. New foam_io.py reads OpenFOAM meshes/fields. | Four lines left the field between them unconstrained; Nusselt needs accurate wall gradients. | Temperature is learned (no collapse); held-out error becomes measurable. |
| 8ff9d1d | Two-phase training: fit the data first (3000 epochs), then ramp in the PDE residuals, wall heat flux and force balance (3000 epochs, ramp 500). | Switching the energy equation on with a random network drove every temperature to T = 0. | Collapse gone. The physics phase is essential: without it dp/dz is 36% off and iso-flux Nu is meaningless. |
| 8ff9d1d | Fourier feature scale 4 -> 1 (low-frequency inputs). | High frequencies add tiny wiggles that ruin second derivatives in the PDEs. | Largest single improvement: Nusselt error fell from tens of % to a few %. |
| 8ff9d1d | One thermal output per (Pr, wall BC) case, each scaled by its own CFD temperature range. | A single Pr-conditioned output under-weighted liquid metal (its errors were 13x smaller in the loss). | Every case fitted equally well. Trade-off: temperature only at the trained Pr values. |
| 8ff9d1d | Integral force balance (G x area = wall shear x perimeter) added as a loss. | The pointwise residual alone did not pin down the pressure gradient. | Learned dp/dz 0.1825 vs CFD 0.1822 (test copy). |
| 8ff9d1d | Loss weights: wall_heat_flux 0.001 -> 0.1, pde_energy 1e-4 -> 3e-4 (chosen from experiments E6 vs E7). | Iso-flux Nusselt was 5-18% off because the wall flux condition was too weak. | Iso-flux Nu within 1.5% of CFD for Pr <= 2. |
| 8ff9d1d | Viscosity nu = 8.0099e-6 -> 7.2653e-6 (= 0.0712/9800, the DNS definition of Re); Nusselt uses Dh = 0.0712 m. | Removes the 'different Dh' caveat: same fluid, same Re and Nu definitions as the DNS. | Your dp/dz 0.18809 -> 0.18202; Nu changed by a few % only. |
| 8ff9d1d | Extractor now also writes cfd_nusselt.csv (CFD's own Nu, DNS Eq. 4-5) and checks dp/dz from the wall shear. | Separates machine-learning error (PINN vs CFD) from RANS error (CFD vs DNS). | Your check: 0.18202 from wall shear vs 0.18202446 in the solver log (5 digits). |
| 8ff9d1d | Evaluation: held-out error table; Nusselt table and bar chart with PINN, CFD and DNS side by side. Notebook, tests and READMEs updated. | Results must show the three levels of comparison separately. | Tests: 5 passed. |
| 8e50dce | Iso-flux wall temperature rebuilt from the stored gradient when OpenFOAM v13 omits the value. | Extractor crashed on your v13 files. | Extraction works on your machine; identical results to the value-based path. |

# 4. CFD vs the two reference papers

Values on the DNS unit-cell path (earlier viscosity, before the change in Section 3). Paper values read by eye from the figures, about +-5-10%; DNS velocities converted from wall units with the local friction velocity (Figs 6-7).

| Quantity | Our CFD | 2023 DNS | 2018 URANS | CFD/DNS | Verdict |
| --- | --- | --- | --- | --- | --- |
| Velocity, subchannel centre | 1.39 | 1.36 | - | 1.02 | Match |
| Velocity, gap centre | 0.553 | 0.85 | 0.55 | 0.65 | Match 2018, low vs DNS |
| Wall shear, gap / 45 deg | 0.29 | 0.69 | - | 0.43 | Low |
| TKE near gap wall | 0.0015 | 0.0128 | - | 0.12 | ~10x low |
| TKE at gap centre | 0.00085 | 0.0101 | - | 0.08 | ~10x low |
| TKE at xi/Dh = 0.6 | 0.0093 | 0.0099 | - | 0.94 | Match |
| TKE at subchannel centre | 0.0044 | 0.0042 | - | 1.05 | Match |
| TKE peak near 45 deg wall | 0.0117 | 0.0219 | - | 0.53 | Low (known SST trait) |
| Pr 0.025: wall heat flux gap/45 | 0.25 | 0.27 | - | 0.93 | Match |
| Pr 1: Theta gap / subchannel centre | 0.41 | 0.67 | - | 0.62 | Low |
| Pr 1: wall heat flux gap/45 | 0.30 | 0.68 | - | 0.43 | Low |
| Pr 2: Theta gap / subchannel centre | 0.51 | 0.76 | - | 0.67 | Low |

### Interpretation

- One cause: the DNS has a gap vortex street (3.7 Hz pulsations) that carries fast fluid and heat through the narrow gap. Steady RANS cannot produce it, so the gap gets too little turbulence, flows too slowly and mixes heat too weakly.

- Model, not set-up: the 2018 URANS with the same k-omega SST model gives the same gap velocity (0.55).

- Liquid metal is the proof: at Pr = 0.025 heat moves by conduction (Pe = 245), so the turbulence error barely matters and the CFD matches the DNS even in the gap. At Pr 1-2 turbulence carries the heat, and the gap error appears. The Pr 1 wall-heat-flux ratio (0.30) tracks the wall-shear ratio (0.29): heat transfer follows momentum transfer.

- 2018 thermal figures (Figs 20, 22) use a different formulation (periodic scaled temperature, no uniform sink) and even disagree with the DNS, so they were used qualitatively only.

## CFD Nusselt numbers on your machine (final, new viscosity)

| Pr | Wall BC | Nu CFD | Nu DNS | CFD vs DNS |
| --- | --- | --- | --- | --- |
| 0.025 | iso-T | 7.70 | 7.54 | +2% |
| 0.025 | iso-flux | 4.88 | 8.85 | -45% |
| 1 | iso-T | 33.32 | 38.74 | -14% |
| 1 | iso-flux | 25.39 | 40.19 | -37% |
| 2 | iso-T | 45.38 | 52.00 | -13% |
| 2 | iso-flux | 36.73 | 52.77 | -30% |
| 7 | iso-T | 74.20 | - | no DNS |
| 7 | iso-flux | 65.60 | - | no DNS |

Iso-flux is far off because the CFD's gap wall runs about 2x hotter than the rod average (a hot spot the DNS does not have), which pulls the mean wall temperature up and Nu down - the same missing gap mixing again. Checks on your run: dp/dz from wall shear 0.18202 = solver 0.18202446; flow converged (1.5e-4 change); thermal converged (0.28%); heat balance 1e-8 except Pr 7 iso-T (0.36%, slightly under-converged).

# 5. The PINN: failure, diagnosis and fix

## 5.1 First full training (your machine, old code)

- Flow: w, nu_t, k matched the CFD lines (errors below 1%).

- Temperature: all 8 cases flat at about 0; data_T, wall_heat_flux and pde_energy losses stuck at exactly their T = 0 values.

- Nusselt: meaningless (e.g. 322 for Pr 0.025 iso-flux vs 8.85 DNS). Apparent near-matches were coincidence.

- dp/dz 0.295 vs CFD 0.188 (+57%); momentum residual never decreased.

## 5.2 Independent test copy

To test fixes without 30-minute trial runs on your laptop, the case was rebuilt here: the same mesh and k-omega SST flow on OpenFOAM v1912, and the 8 temperature fields from a separate finite-volume solver (checked against OpenFOAM's laplacianFoam to 0.02%). It matched your data: dp/dz 0.1883 vs 0.1881; iso-T minima -1.036 / -6.97 / -9.73 / -19.41 vs your -1.04 / -6.98 / -9.74 / -19.4. The old code reproduced your failure on it (dp/dz 0.29, temperature not learned).

## 5.3 Experiments (test copy)

| Run | Set-up | Held-out w | Held-out T | Nu vs CFD | dp/dz (CFD 0.1822) |
| --- | --- | --- | --- | --- | --- |
| E1 | Full-field data only, 1500 ep. | - | 6.8% | -26% to +1300% | 0.134 |
| E2 | Data + physics, Fourier scale 4 | - | 5.4% | -58% to +34% | 0.169 |
| E3 | Data only, 3000 ep. | - | 4.0% | -535% to +124% | 0.134 |
| E4 | + wall data, Fourier 1, physics | 0.9% | 1.7% | -2% to +18% | 0.183 |
| E5 | Same as E4 without physics | 0.7% | 0.9% | iso-flux negative | 0.117 |
| E6 (chosen) | E4 + stronger wall-flux weight, 6000 ep. | 0.5% | 1.4% | within 1.5% (Pr<=2), Pr 7 flux +9% | 0.1825 |
| E7 | E6 with 10x physics weights | 0.6% | 2.8% | within 3% (Pr<=2), Pr 7 +8/+12% | 0.1826 |

Key lessons for the methodology section: (1) the physics phase is what makes the pressure gradient and iso-flux results right - data alone cannot; (2) high-frequency input features must be avoided when the loss contains second derivatives; (3) the PDE must be introduced after the data fit, not from a random start; (4) stronger physics weights lower the residuals but worsen the data fit - E6 is the balance.

## 5.4 Final result on the test copy (E6, 6000 epochs, ~25 min)

| Measure | Result |
| --- | --- |
| Held-out error (RMSE / range) | w 0.49%, nu_t 1.1%, k 1.7%, T 0.3-1.4% per case (Pr 7 iso-T 3.3%) |
| Learned dp/dz | 0.1825 vs CFD 0.1822 (+0.2%) |
| Nusselt, PINN vs CFD | Pr 0.025-2: within 1.5% (all six DNS cases); Pr 7: +1.4% iso-T, +8.7% iso-flux |
| Momentum residual (RMS / G) | 0.15 (was ~1 in the first run) |
| Energy residual (RMS / sink) | 0.4-1.9: reduced but not zero - disclose |

# 6. Honest limitations to state in the paper

- Steady RANS (k-omega SST): no gap vortex street, no secondary flow, constant Pr_t = 0.9. This causes the gap and iso-flux differences from the DNS.

- Unit cell with symmetry planes, not the DNS's confined six-rod domain; the DNS central cell's local bulk velocity is not published.

- Paper values in Section 4 are read by eye (+-5-10%) until the figures are digitized.

- PINN results in Section 5.3-5.4 are from the test copy; the final numbers must come from your own run.

- PDE residuals are reduced, not zero; Pr 7 is the least accurate case and has no DNS counterpart.

- Single Reynolds number (9800); temperatures only at the trained Pr values.

# 7. Next steps

- Run the updated notebook top to bottom (Kernel > Restart first; about 30-60 min of training).

- Record: learned dp/dz (expect ~0.182), held-out error table, Nusselt table (PINN_vs_CFD should be within ~2%).

- Digitize DNS Figs 7 and 12a (then 6 and 11a) into digitized_data/ for the remaining independent comparisons.

- Optional: run the thermal case longer to close the Pr 7 heat balance (0.36%).

- Update the older project-plan PDF and professor docx, which still describe the superseded set-up.